A Gamified Smart Cane for

Orientation and Mobility Training

Advanced Mechatronics - ROB-GY 6103

Team 5

Bao Do, Anudamodaran Kumarakurup, Tejas Mathur Sriganesh

New York University

# Table of Content

Table of Content	1

1. Abstract	2

2. Introduction	2

3. Problem Statement	3

4. Solution Approach	3

4.1 Hardware	3

4.2 Firmware Architecture	4

4.3 Mode Selection	4

4.4 Game Modes — Pillars 1 and 2	4

4.5 Scoring	5

4.6 Haptic Vocabulary	5

4.7 Game Modes — Pillars 3 and 4	6

4.8 Computer Vision Pipeline	7

4.9 Laptop-to-Propeller Communication	7

5. Test Procedure and Results	8

5.1 Hardware Verification	8

5.2 Mode Testing	8

5.3 Software Prototype Testing	9

6. Discussion	10

7. Conclusions	10

8. Appendix	10

A. Pin Assignment Table	10

B. Key Firmware Constants	11

C. File Structure	12

# 1. Abstract

GuidePlay is a white cane with sensors that turn orientation and mobility (O&M) training into a haptic feedback game that operates in real time. An IMU, three ultrasonic sensors, and three vibration motors controlled by a Parallax Propeller Activity Board are all integrated into the cane. Sweeping, Touch Rhythm, Shore Lining, and Ground Reading are the four training modes that focus on the fundamental cane skills needed for self-navigating. Memory Run and Cartographer are two more modes that add computer vision AR overlays and a Bluetooth module to the system, improving entertainment while training. A haptic result signal is displayed at the end of each 30-second session. No screen, no numbers, and no cognitive disruption to physical practice are necessary for this system.

# 2. Introduction

White cane technique is the foundation of independent navigation for visually impaired individuals. Consistent arc sweeping, rhythmic two- and three-point touch, shore lining, and ground surface reading are some of the specific techniques that must be mastered for proficiency. Traditionally, one-on-one sessions with qualified O&M specialists in specialized training gyms are used to teach these skills. This resource-intensive method limits access and introduces trip hazards during the learning process.

Environmental detection and obstacle avoidance have been the focus of recent smart cane research, which routes navigation commands to the user via audio alerts. Although this method is technologically fascinating, interviews show that it produces "continuous partial attention" that leads to fatigue and prevents the formation of spatial memory. "I want to know I did it right, not that the device saved me" is how users expressly reject devices that feel like automation or surveillance. The conflict between environmental detail and confidence is still unsolved in assistive technology today.

GuidePlay adopts a different strategy, teaching navigation instead of automating it. The technology, which was created through collaborative design with blind and low-vision people, enhances rather than replaces the analog cane experience. Haptic vibration is the only way that these game grades are returned. This eliminates the cognitive strain of audio alerts while offering the structured feedback loop that is often offered by trainers. By providing instructor vision without interfering with the user's proprioceptive focus, a ceiling camera and Python-based augmented reality overlay ensure that the technology stays "with, not merely for" the user.

# 3. Problem Statement

Three specific gaps motivate this project:

- The Technique Gap: Instead of prioritizing skill development, current smart cane technology concentrates on destination assistance. Cane mechanics, such as arc width, sweep tempo, tip contact consistency, and wall proximity, cannot be graded in real time during a training session by any commercially available technology.

- The Infrastructure Gap: O&M training facilities need specialized personnel, specialized physical space, and specially designed obstacle installations. This hinders independent, self-directed repetition outside of official teaching and restricts practice to regular periods.

- The Feedback Gap: Continuous partial attention is produced by audio-based smart cane feedback, which weakens the development of spatial memory and wears the user out. To maintain the necessary sensory concentration for navigation, effective training feedback must be provided via the haptic channel. Feedback must also be given right away; delays longer than 100ms will disrupt the learning loop.

# 4. Solution Approach

GuidePlay utilizes an integrated hardware and software architecture to fill all three gaps. The sensor and actuator package is carried by the actual cane. The computer vision pipeline, AR display for instructors, and game engine are all run on a companion laptop.

## 4.1 Hardware

Each vibration motor is driven through a 2N2222 NPN transistor with a 1kΩ base resistor for back-EMF protection. The IMU communicates over I2C on the Activity Board.

## 4.2 Firmware Architecture

Shared volatile integer variables in global memory are used for all inter-cog communication. The firmware does not employ floating-point arithmetic, dynamic memory allocation, or EEPROM access. To prevent wraparound issues, sensor timestamps employ unsigned integer arithmetic with the Propeller's 32-bit CNT register.

The firmware performs a 3-second calibration procedure at the beginning of each session, averaging 50 IMU readings to create individual baselines for accelerometer resting values and gyro zero-rate. To account for differences in user physique and mounting angle, all following threshold comparisons are based on these baselines rather than hardcoded numbers.

## 4.3 Mode Selection

The IMU gyroscope detects wrist-twist motions to choose the mode and difficulty before each session, doing away with the necessity for buttons. The next setting is reached with a 90-degree clockwise twist. A haptic buzz verifies the choice when the cane is held for one second. Beginner, Intermediate, and Advanced are the three difficulty levels that the same mechanic goes through.

## 4.4 Game Modes — Pillars 1 and 2

Four modes run entirely on the Propeller firmware without laptop involvement:

Using the IMU gyroscope, sweeping grades arc width and tempo. Each half-sweep ends with a direction reversal (zero crossing of the gyro rate). Arc width is approximated by the peak gyro rate during the half-sweep. When the arc drops below the threshold, the side motor on the narrow side emits a gentle buzz, while the front motor activates for tempo drift.

Depending on the level of difficulty, Touch Rhythm plays a metronome beat at 60, 75, or 90 BPM. Every tap impact spike is detected by the IMU accelerometer. A third probe tap is introduced at the Intermediate and Advanced levels; on the third beat, the front ultrasonic must locate a simulated landmark within 20cm. While missing beats result in a front motor warning, confirmed taps generate matching motor pulses.

Shore lining tracks left ultrasonic proximity to a virtual wall. A target distance band (30 cm by default, with a tolerance of ± depending on difficulty) is maintained by the player. A harsh buzz is triggered by approach, whereas a mild left motor pulse is triggered by a drift. The player must simultaneously center between two walls when the right ultrasonic activates Advanced difficulty.

For tip lift events, Ground Reading tracks IMU Z-axis acceleration. All three motors fire in a stutter pattern when the lift exceeds the threshold. The front ultrasonic detects curbs by looking for abrupt reductions in distance. The touch-and-slide gesture, which requires a minimum 150 ms contact hold and a gyro-detected lateral slide verified by a front motor pulse, is introduced at Intermediate and Advanced levels.

## 4.5 Scoring

Each session runs for 30 seconds, divided into 300 scoring intervals of 100ms each. The game determines whether all active method metrics are concurrently inside tolerance at each interval. The number of passing intervals determines the final score:

| Result | Intervals in Tolerance | Percentage | Haptic Signal |
| --- | --- | --- | --- |
| Good | 210 or more | 70 %+ | 3 long pulses |
| Partial | 150 – 209 | 50 – 69 % | 2 long pulses |
| Try Again | Fewer than 150 | < 50 % | 1 long pulse |

Table 1. Session scoring thresholds.

## 4.6 Haptic Vocabulary

A consistent vibration vocabulary is used across all modes, so the player builds intuition without cognitive translation:

| Pattern | Duration | Meaning |
| --- | --- | --- |
| Soft single pulse | 80ms | Correct action — confirmation |
| Hard single pulse | 150ms | Warning — near boundary or hazard detected |
| Triple stutter | 3 × 50ms | Critical error — tip lifted from ground |
| Calibration buzz | 500ms hold | System ready — hold neutral position |

Table 2. Haptic feedback vocabulary.

## 4.7 Game Modes — Pillars 3 and 4

Two advanced modes use the laptop game engine alongside the Propeller firmware, communicating over Wi-Fi HTTP. The player is blindfolded throughout — audio tones play through an earpiece for game events, and the vibration motors activate exclusively for boundary warnings. The TV display is for observers only.

Memory Run is a route recall test. During the LEARN phase, the player moves through checkpoints one at a time. Each checkpoint triggers an audio tone through the earpiece, rising in pitch across successive checkpoints. During the RECALL phase, the player retraces the route from memory with no audio guidance. The ceiling camera tracks the cane's color marker and scores the attempt based on path deviation and checkpoint order. At Level 3, all feedback including boundary warnings is disabled.

Cartographer is an object location recall test. During the EXPLORE phase, the player moves freely within the workspace. When the cane marker comes within range of a virtual obstacle, a discovery tone plays through the earpiece. The player has a fixed time (15–25 seconds depending on level) to find as many objects as possible. During the QUIZ phase, an audio prompt names a target and the player navigates to its remembered location. A confirmation tone plays on arrival. Random obstacle placement on every run prevents memorization.

In both modes, boundary feedback keeps the player within the workspace. When the cane marker enters the warning zone, the laptop sends HTTP commands to the Propeller driving the vibration motors in a slow stutter. At the boundary itself, the motors switch to a rapid stutter. The earpiece simultaneously plays warning beeps in the warning zone and a continuous drone at the boundary. All feedback stops immediately when the player returns to the safe zone.

For observers, the TV renders a live overhead view. Memory Run overlays the taught path in gold and recalled path in green. Cartographer uses a fog-of-war effect that reveals areas as the player explores and hides objects during the quiz phase.

## 4.8 Computer Vision Pipeline

The laptop runs a Python/OpenCV application to track the cane from a ceiling-mounted USB camera. A brightly colored tape marker on the cane serves as the tracking target — chosen over ArUco fiducial markers because HSV color detection has no minimum marker size, allowing a small piece of tape to work reliably at ceiling height.

Before gameplay, the system goes through calibration and workspace setup. During calibration, the user clicks on the marker in the camera feed to sample a 5×5 pixel region, computing the median HSV values. Six sliders auto-adjust to a tolerance range around the sampled color, and a live binary mask confirms detection. During workspace setup, the user draws a rectangle on the camera feed defining the play area — all game objects spawn within this boundary.

During tracking, each frame is converted to HSV, thresholded with inRange, cleaned with erosion and dilation, and the largest contour's centroid is computed via image moments, producing the cane's (x, y) position at approximately 30 FPS. The game engine uses this position for checkpoint proximity, obstacle discovery, path recording, and boundary distance calculations. The AR overlay — fog-of-war, checkpoints, path trails, scores, and boundary indicators — is composited onto the camera frame and output to the TV via HDMI.

## 4.9 Laptop-to-Propeller Communication

The laptop game engine communicates with the Propeller microcontroller over a Wi-Fi HTTP link. The Propeller runs a lightweight web server at a static IP address (192.0.0.4:8080). The laptop sends commands as HTTP GET requests with a command parameter — "1" activates the vibration motors, "0" deactivates them. This wireless approach was chosen over USB serial to avoid restricting the player's movement with a physical cable and to eliminate platform-specific serial port configuration.On the laptop side, commands are sent using the Python requests library in background threads with a 0.3-second timeout. The game engine never waits for a response before proceeding to the next frame. If the Propeller is unreachable due to Wi-Fi dropout or the server not running, the request silently times out and the game continues without haptic feedback, degrading gracefully rather than freezing.On the Propeller side, the web server runs in a dedicated cog, isolated from sensor reading and game logic. It parses the command parameter, sets or clears the motor output pins, sends a short acknowledgment, and closes the connection immediately.

The boundary warning system drives this link at its highest frequency. During normal gameplay, commands are only sent on game events. During boundary approach, the laptop sends alternating on/off commands at a rate determined by proximity — every 0.6 seconds in the warning zone (40–70 pixels from the edge), accelerating to every 0.2 seconds at the boundary itself (under 40 pixels). This produces the slow and fast stutter patterns the player feels through the cane. All timing is controlled by the laptop, keeping game logic centralized and reducing the Propeller firmware to a simple command executor.

# 5. Test Procedure and Results

## 5.1 Hardware Verification

Prior to integration, each hardware component was checked separately. By flipping each propeller output pin and using an oscilloscope on the transistor collector to verify motor response, the motor circuit was tested. By printing distance readings to the SimpleIDE terminal at known distances of 10, 30, and 60 cm, the ultrasonic sensors were validated; recorded values were within ± 2 cm of ground truth. The I2C register (anticipated 0x68) and live gyroscope and accelerometer data during intentional cane motions were used to validate the IMU.

During ultrasonic testing, a crucial problem was discovered: the Propeller's pause() function functions in milliseconds, resulting in a 10ms trigger pulse rather than the necessary 10 µs. In order to prevent overflow, this was fixed by substituting CNT-based waitcnt() calls with unsigned integer arithmetic for pause(). Additionally, the 32-bit counter rollover at roughly 53 seconds of runtime was handled by updating all CNT elapsed-time computations to utilize unsigned subtraction.

## 5.2 Mode Testing

Each game mode was tested in isolation using hardcoded mode and difficulty values to bypass the selection interface. Key observations:

- Sweeping: Since the gyro rate is by definition close to zero at the zero crossing, arc intensity was first measured there. By monitoring the peak gyro rate in between crossings and assessing it at the crossing event, this was fixed. During physical testing, the arc threshold was adjusted empirically.

- Ground Reading: Three I2C transactions per 100ms loop iteration were caused by a duplicate IMU read inside the touch-slide detection block, which periodically caused the bus to stall. The problem was fixed by combining all sensor readings into a single block at the top of the loop. To produce the proper downward spike when the tip touches a surface, touch detection was adjusted from the X-axis (lateral) to the Z-axis (vertical).

- Shore lining: At a goal distance of 30 cm, threshold values were verified to be functional.

- Touch Rhythm: By displaying the raw accelerometer readings during intentional taps on a hard surface, the tap axis and threshold were confirmed.

## 5.3 Software Prototype Testing

The color tracker was validated across lighting conditions with orange and yellow tape markers. During testing, a bug was found where the S High and V High sliders were capped at 179 instead of 255 — the trackbar code checked "H" in slider_name, which matched "S High" and "V High" due to the H in "High." The fix changed the condition to slider_name.startswith("H"). After correction, full-range masking worked as expected.

The Propeller HTTP communication was tested for latency impact. The non-blocking architecture introduced no visible lag even when the server was unreachable. Motor stutter transitions between slow and fast responded within one frame of the boundary state change.

The Memory Run prototype was tested with three to five checkpoints. Phase transitions, audio tones, and path overlay rendering (gold for learned, green for recalled) all functioned correctly. The Cartographer prototype was tested with three to seven randomly placed objects. 	Fog-of-war rendering darkened only the workspace area; objects appeared at the 60-pixel discovery radius with audio confirmation, and the 45-pixel quiz arrival radius was tuned through playtesting to balance difficulty for a blindfolded player.

# 6. Discussion

One significant addition was the wrist-twist selecting interface. By repurposing the IMU that is currently utilized for technique grading, the removal of buttons maintains interaction within natural cane gestures without requiring additional hardware.

Testing verified computer vision decisions: HSV color tracking over ArUco markers overcomes minimum size restrictions; colorful tape can be recalibrated in a matter of seconds in any type of lighting. Across environments, the two-click calibration method demonstrated resilience.

The threshold constants are still tentative. In order to mimic the motor patterns of experienced cane users, the lift threshold, tap threshold, and arc intensity values were adjusted using sighted testers and need to be validated with visually impaired participants.

# 7. Conclusions

GuidePlay shows that low-cost embedded hardware can provide real-time technique grading for white cane O&M training without the need for screen-based feedback. During live sessions, the four firmware-only training modes—Sweeping, Touch Rhythm, Shore lining, and Ground Reading—are fully functional and generate physically significant haptic input. Memory Run and Cartographer, two computer vision-augmented modes, expands the system's capabilities to handle cognitive training tasks that the firmware cannot handle on its own. Future work should prioritize user testing with the target population, threshold calibration from empirical data, and integration of the firmware and laptop game engine into a unified session flow with real-time serial communication.

# 8. Appendix

## A. Pin Assignment Table

| Pin | Signal | Component |
| --- | --- | --- |
| P0 | MOT_L | Left vibration motor (via 2N2222) |
| P1 | MOT_R | Right vibration motor (via 2N2222) |
| P2 | MOT_F | Front vibration motor (via 2N2222) |
| P3 | F_TRIG | Front ultrasonic trigger |
| P4 | F_ECHO | Front ultrasonic echo |
| P5 | L_TRIG | Left ultrasonic trigger |
| P6 | L_ECHO | Left ultrasonic echo |
| P7 | R_TRIG | Right ultrasonic trigger |
| P8 | R_ECHO | Right ultrasonic echo |
| P9 | I2C_SCL | IMU clock |
| P10 | I2C_SDA | IMU data |
| P14 | HC05_txd | bluetooth module tx |
| P15 | HC05_rxd | bluetooth module rx |

Table A1. Propeller Activity Board pin assignments.

## B. Key Firmware Constants

| Constant | Value | Description |
| --- | --- | --- |
| GYRO_SCALE | 131 LSB/°/s | MPU-6050 default sensitivity at ±250 °/s range |
| TWIST_THRESH | 60° | Minimum integrated angle for mode/difficulty advance |
| NEUTRAL_RATE | 5 °/s | Maximum gyro rate considered stationary |
| CONFIRM_MS | 1000ms | Hold duration required to confirm selection |
| SESSION_MS | 30000ms | Session length |
| LOOP_INTERVAL_MS | 100ms | Scoring interval duration |
| SCORE_INTERVALS | 300 | Total scoring intervals per session |
| LIFT_THRESHOLD | 500 LSB | Accel Z delta that counts as a tip lift event |
| CURB_THRESHOLD | 10 m | Front ultrasonic drop that counts as a curb |
| TAP_THRESHOLD | 300 LSB | Accel X magnitude that counts as a tap |
| TOUCH_HOLD_MS | 150ms | Minimum tip contact for touch-slide gesture |
| WALL_TARGET_CM | 100 cm | Default target wall distance for shorelining |

Table B1. Key firmware configuration constants.

## C. File Structure

- Pathfinder.c — startup, IMU init, mode/difficulty selection, calibration, session dispatch

- config.h — all pin assignments, threshold constants, difficulty arrays

- imu.c / imu.h — I2C init, register reads, 6-axis extraction, calibration

- ultrasonic.c / ultrasonic.h — CNT-based trigger/echo timing, cm conversion

- motors.c / motors.h — PWM output, buzz pattern functions

- modes.c / modes.h — four mode logic functions, run_mode() dispatcher